KULLIYYAH OF ENGINEERING

MECHATRONICS SYSTEM INTEGRATION (MCTA 3203)

SEMESTER 2, 2025 / 2026

WEEK 3 : SERIAL COMMUNICATION
SECTION 2, GROUP 4

INSTRUCTORS : ZULKIFLI BIN ZAINAL ABIDIN & WAHJU SEDIONO

Date of Submission: 1st April 2026

| NO | NAME | MATRIC NUMBER |
| --- | --- | --- |
| 1 | Hannah Irdeena binti Azrulhafiz | 2410018 |
| 2 | Azzam Azhari Noureldein Elamin | 1827295 |
| 3 | Kamarul Zafri bin Kamarul Ariffin | 2323189 |

# ABSTRACT

This experiment investigates the practical use of serial communication between an Arduino microcontroller and a computer interface, with Python employed for both data acquisition and device control. The study was conducted in two main sections. The first involved acquiring analog signals from a potentiometer via the Arduino’s serial interface and displaying the corresponding readings in a Python console. The second focused on controlling a servo motor, where its angular position was adjusted according to user-defined commands sent from Python to the Arduino.

To enhance the system’s functionality, several improvements were incorporated. These included real-time servo control based on potentiometer input, LED indicators to signal threshold levels, and live graphical representation of sensor data using the Matplotlib library. Overall, the experiment demonstrates effective integration of hardware and software, highlighting how serial communication enables interaction between embedded systems and desktop applications. It also showcases the capability of combining Arduino and Python for real-time monitoring, automated control, and responsive feedback in mechatronic and data-driven applications.

# TABLE OF CONTENTS

| NO. | TOPIC |
| --- | --- |
| 1. | INTRODUCTION |
| 2. | MATERIALS AND EQUIPMENT |
| 3. | EXPERIMENTAL SETUP |
| 4. | METHODOLOGY |
| 5. | DATA COLLECTION |
| 6. | DATA ANALYSIS |
| 7. | RESULTS |
| 8. | DISCUSSION |
| 9. | CONCLUSION |
| 10. | RECOMMENDATIONS |
| 11. | REFERENCES |
| 12. | APPENDICES |

## 1.0 INTRODUCTION
	Serial communication is a fundamental and widely utilized method for data exchange between microcontrollers and computers. It operates by transmitting data one bit at a time through a communication channel, providing a simple and reliable means of connecting electronic devices. This approach is particularly advantageous in embedded systems and mechatronic applications, where dependable real-time data transfer and control are essential.

## In this experiment, an Arduino board is used to interface with a potentiometer, a variable resistor that produces a changing voltage output based on its position. As the potentiometer is adjusted, it generates an analog voltage signal, which is read by the Arduino using its built-in Analog-to-Digital Converter (ADC). The ADC converts the analog signal into a corresponding digital value that can be processed by the system.

## The acquired digital data is then transmitted from the Arduino to a computer via a serial connection. A Python program on the computer serves as the receiving interface, capturing and processing the incoming data stream and displaying the values in real time. This enables continuous monitoring of changes in the potentiometer’s position, offering immediate feedback on system behavior.

## Overall, this setup demonstrates the effective integration of hardware and software in mechatronic systems. It illustrates how microcontrollers can be combined with high-level programming languages to develop interactive and responsive applications. Such serial communication techniques are widely applicable in prototyping, education, and industrial automation, forming a core component of modern control and monitoring systems.

## PART A
2.0 MATERIALS & EQUIPMENT

1. Arduino Uno (or compatible board)

2. Potentiometer

3. Breadboard

4. Jumper wires

5. Arduino IDE and PyCharm

3.0 EXPERIMENTAL SETUP

The setup for this experiment is made up of both hardware and software components which operate with each other, enabling serial communication between the Arduino board and Python. It can be divided into four main sections :

1. Potentiometer Input
	The potentiometer employed in this experiment serves as an analog input device that produces a variable voltage corresponding to the position of its rotating knob. It has three terminals: two outer terminals connected to 5 V and ground (GND), and a central wiper terminal linked to the Arduino Uno’s analog input pin A0.
When the knob is rotated :

● The wiper moves along the internal resistive track, dividing the total resistance proportionally.

● This produces an output voltage between 0V and 5V, representing the position of the potentiometer.

● The Arduino’s Analog-to-Digital Converter (ADC) converts this voltage into a 10-bit digital value ranging from 0 (0V) to 1023 (5V).

As the potentiometer is rotated clockwise, the analog reading increases, whereas rotation in the counterclockwise direction results in a decrease in the measured value. These readings are transmitted to the computer via serial communication and subsequently visualized using Python.

This process illustrates how analog signals from sensors or variable components can be converted into digital data and communicated for monitoring and control applications.

2. Assembly and Wiring Checks

Before running the program, proper circuit assembly and verification are crucial to ensure accurate readings and prevent hardware issues.

a. Assembly Steps

1. Mount the Arduino Uno on a flat, static-free surface.

2. Place the potentiometer on the breadboard for stable connections.

3. Connect the left terminal of the potentiometer to the 5V pin on the Arduino using a red jumper wire.

4. Connect the right terminal of the potentiometer to the GND pin using a black jumper wire.

5. Connect the middle terminal (wiper) of the potentiometer to the A0 analog input pin using a yellow jumper wire.

6. Use a USB cable to connect the Arduino to the computer for both power supply and data communication.

b. Wiring Checks

After assembly, verify the following :

● All jumper wires are firmly inserted and match the correct Arduino pins.

● The potentiometer terminals are connected to the correct power rails (5V and GND).

● The A0 pin connection is not loose or shorted.

● No wires are crossing or touching each other, which may cause short circuits.

● The USB connection is recognized by the computer (visible as a COM port in Arduino IDE).

A proper wiring check ensures reliable analog readings and stable serial communication between the Arduino and the computer.

## 3. Software and Initial Tests

## a. Arduino Program

## The Arduino sketch is written and uploaded using the Arduino IDE. The program performs the following tasks :

## void setup() {

## Serial.begin(9600);

## }

## void loop() {

## int potValue = analogRead(A0);

## Serial.println(potValue);

## delay(1000); // add a delay to avoid sending data too fast

## }

## b. Python Program

## On the computer side, Python is used to read and display data from the Arduino. The PySerial library establishes communication, while Matplotlib provides real-time visualization.

## import serial

## # -- Replace 'COMx' with your Arduino's serial port

## ser = serial.Serial('COMx', 9600)

## try:

## while True:

## pot_value = ser.readline().decode().strip()

## print("Potentiometer Value:", pot_value)

## except KeyboardInterrupt:

## ser.close()

## print("Serial connection closed.")

## 4. Troubleshooting
	During the experiment, issues may arise from wiring errors, port misconfigurations or software bugs. The following list shows the common occurring problems and their solutions :

## Problems

- No readings on the Serial Monitor

- Unstable or fluctuating readings

- Constant “0” or “1023” reading

- Python program does not receive data

- Graph freezes or lags

Possible Causes

- COM port not selected or wrong baud rate

- Loose connections or noisy analog input

- Potentiometer terminal miswired

- Wrong COM port or permission error

- Missing .pause() or sampling rate is too high

Solutions

- Check the correct COM port in Arduino IDE and ensure baud rate = 9600

- Reconnect wires securely and use shorter jumper wires

- Verify wiper pin (middle) is connected to A0

- Update the correct COM port in Python code (‘COM4’)

- Add a small delay or adjust sampling interval

## 4.0 METHODOLOGY

## The potentiometer was connected to the Arduino according to the specified wiring configuration, ensuring correct connections to 5 V, ground (GND), and analog input pin A0.

## An Arduino program was uploaded to read the analog input using the analogRead(A0) function. The measured values were transmitted via serial communication using Serial.println(value) at a baud rate of 9600.

## A Python script utilizing the pyserial library was developed to establish communication through the appropriate COM port and receive the incoming serial data from the Arduino. The matplotlib library was employed to visualize the potentiometer readings in real time.

## After both the Arduino and Python programs were executed, the potentiometer was gradually adjusted to vary its resistance. The resulting changes in analog values were transmitted to the Python interface and displayed dynamically in graphical form.

## Observations were recorded to examine the relationship between the potentiometer position and the corresponding analog values, which ranged from 0 to 1023. The results confirmed the reliability and accuracy of serial communication between the Arduino and Python systems.

## 5.0 DATA COLLECTION

The experiment collected data through a potentiometer which transmitted its output to the Arduino system through the analog input A0. The potentiometer generated analog voltage output which extended from 0 V to 5 V and this output was transformed into digital signals through the 10-bit ADC of the Arduino which had a measurement range from 0 to 1023.

The Arduino system continuously monitored the potentiometer output while sending structured data to Python through serial communication which operated at a speed of 9600 baud. The transmitted data included:

- Output voltage (Vout)

- ADC value

- Calculated resistance

The Python script received this serial data line-by-line, decoded it, and extracted the relevant parameters. The Arduino received commands from Python which controlled the LEDs by sending "LED ON" or "LED OFF" signals based on the received ADC values.

## 6.0 DATA ANALYSIS

The analysis required interpreting the potentiometer readings to establish control actions.

The Arduino system used its first component to convert raw ADC values into voltage through its second component.

The system calculated wiper resistance through its mechanism which divides voltage according to proportionality.

The Python program extracted numeric data from incoming string data which included voltage and ADC and resistance measurements. The system operated its main control logic through a threshold-based mechanism.

The system activated the LED when ADC values exceeded 512 which represented half of the maximum range. The system deactivated the LED when ADC values dropped to 512 or below.

The system operated through a basic feedback control mechanism which used analog input to control digital output.

## 7.0 RESULTS

## The system successfully demonstrated bidirectional serial communication between Arduino and Python.

## The ADC and voltage values changed accordingly and were correctly displayed in Python when the potentiometer was turned.

## The LED turned ON when the potentiometer value exceeded the midpoint.

## The LED turned OFF when the value dropped below the threshold

## The system demonstrated real-time response because it confirmed accurate data transmission and correct parsing and effective control logic.

## PART B

2.0 MATERIALS & EQUIPMENT

1. Arduino board

2. Servo motor

3. Jumper wires

4. Potentiometer

5. USB cable for Arduino

6. Computer with Arduino IDE and Python installed

3.0 EXPERIMENTAL SETUP

The experimental setup for this lab consists of both hardware and software components that work together to enable serial communication between the Arduino board and a computer running Python.

1. Servo Motor Input

1. The servo motor was connected to the Arduino Uno to enable controlled angular motion through pulse-width modulation (PWM) signals.

2. The signal wire of the servo was attached to digital pin 9, which supports PWM output and allows precise control of the servo’s rotational angle.

3. The power wire (VCC) of the servo was connected to the 5V output pin on the Arduino to supply the necessary operating voltage.

4. The ground wire (GND) of the servo was linked to one of the Arduino’s GND pins to complete the circuit and ensure a stable reference for current flow.

5. The Arduino Uno was connected to the computer via a USB cable, which served as both the power supply and the data communication channel for serial exchange between the Arduino and Python.

6. Through this serial interface, Python transmitted angle commands to the Arduino, which

interpreted the signals and adjusted the servo’s shaft position accordingly.

7. This configuration allowed real-time servo motor control, enabling smooth and accurate motion based on user input from the Python program.

8. To ensure consistent performance, all jumper wires were secured, connections were verified, and the circuit layout was arranged to minimize signal interference and power fluctuations.

2. Software and Initial Tests

a. Arduino Program

#include <Servo.h> Servo myservo; // to control a servo int pos = 0; // variable to store the servo position void setup() { Serial.begin(9600); myservo.attach(9); // attaches the servo on pin 9 } void loop() { // Check if data is available to read from Python if (Serial.available() > 0) { // Read the incoming integer int angle = Serial.parseInt(); // Ensure the angle is within safe servo limits if (angle >= 0 && angle <= 180) { myservo.write(angle); delay(15); } } }

b. Python Program

On the computer side, Python is used to read and display data from the Arduino. The PySerial library establishes communication, while Matplotlib provides real-time visualization.

import serial """ Define the serial port and baud rate (adjust the port COMx as per your Arduino, e.g., 'COM3' or 'COM7') """ ser = serial.Serial('COMx', 9600) try: while True: angle_input = input("Enter servo angle (0-180 deg, or 'q' to quit): ") if angle_input.lower() == 'q': break try: angle = int(angle_input) if 0 <= angle <= 180: # Send the servo's angle to the Arduino with a newline ser.write((str(angle) + '\n').encode()) else: print("Angle must be between 0 and 180 degrees.") except ValueError: print("Invalid input. Please enter a valid number or 'q' to quit.") except KeyboardInterrupt: pass # Handle keyboard interrupt (CTRL+C) finally: ser.close() # Close the serial connection print("Serial connection closed.")

3. Troubleshooting

During the experiment, several common issues were identified and addressed as follows:

1. No Servo Movement:

○ Cause: Incorrect servo wiring or insufficient power supply.

○ Solution: Verified the signal wire connection to the correct PWM pin and ensured the servo’s power line was properly connected to the 5V pin.

2. Unstable or Jittery Servo Motion:

○ Cause: Electrical noise or fluctuating analog readings from the potentiometer.

○ Solution: Added a short delay in the Arduino loop and confirmed stable wiring to minimize signal interference.

3. No Serial Data on Python Terminal:

○ Cause: Wrong COM port selection or mismatched baud rate.

○ Solution: Ensured the Python serial port matched the Arduino COM port and that both used the same baud rate (9600).

4. LED Not Responding to Signal:

○ Cause: Incorrect logic in the Arduino code or wrong pin assignment.

○ Solution: Reviewed the code logic and verified LED pin mapping in both circuit and program.

## 4.0 METHODOLOGY

## System Initialization :

## The Arduino board was connected to the computer via a USB cable.

## The Arduino IDE was used to upload the program responsible for reading potentiometer inputs and controlling the servo motor position.

## Potentiometer Input Control :

## The potentiometer was manually adjusted to vary its resistance, resulting in changes to the analog voltage at pin A0.

## The Arduino processed this input and mapped it proportionally to a corresponding servo angle, causing the servo motor to rotate accordingly.

## Data Transmission and Visualization :

## The Arduino continuously transmitted potentiometer readings to the computer through serial communication.

## A Python script received the data, displayed it in the terminal, and plotted it dynamically using Matplotlib for real-time visualization of signal changes.

## LED Feedback (Enhancement) :

## An LED indicator was implemented to illuminate when the potentiometer reading or servo position exceeded a predefined threshold, providing a visual representation of system response.

## Testing and Verification :

## The system was evaluated by adjusting the potentiometer across its full operating range while observing the corresponding responses of the servo motor and LED indicator.

## 5.0 DATA COLLECTION

The experiment gathered data from two sources which depended on the specific task being performed.

Task (Manual Control):

- User input from Python (servo angle values between 0° and 180°)

Task 2 (Real-Time Control):

- Potentiometer readings from Arduino (analog input A0), which were mapped to servo angles

The Arduino performed continuous operations to accomplish Task 2 by:

- Reading potentiometer values (0–1023)

- Mapping the values to servo angles (0°–180°)

- Sending the angle values to Python through serial communication at 50 ms intervals

The Python script collected these angle values and stored them for real-time plotting using matplotlib.

## 6.0 DATA ANALYSIS

The analysis focused on mapping input values to servo motion and visualizing system behavior.

- Angle Mapping

The Arduino converted potentiometer readings into angles using:

Angle = map(0-1023→0-180∘)

- LED Logic

A threshold condition was applied:

- Angle > 90° → LED ON

- Angle ≤ 90° → LED OFF

Visualization

The Python script created a graph which showed the relationship between time and servo angle at each sample number. The graph updates in real-time to show current changes in potentiometer input.

Control Mechanism

The system received a stop command through key 'x' which enabled safe shutdown. The Arduino stopped servo operation and turned off the LED when it received the command.

## 7.0 RESULTS

## The servo motor showed precise movement control which responded to both manual movements and real-time input signals.

## In manual mode, entering angle values in Python caused the servo to move to the specified position.

## In real-time mode, the system allowed users to control the servo angle between 0° and 180° through smooth potentiometer rotation.

## The LED turned ON when the angle exceeded 90°, providing clear visual feedback.

## The Python plot continuously updated to display real-time changes in servo angle through a live graph.

## The stop mechanism worked correctly, safely halting the system and preventing unwanted motion.

## The experiment proved successful because it showed how different components worked together to achieve proper sensing actuation and visual display while maintaining effective serial data transfer between Arduino and Python.

## 8.0 DISCUSSION

The experiment demonstrated bidirectional serial communication between an Arduino and computer interface. In Experiment 1, it is demonstrated analog data from a potentiometer digitized and transmitted to a Python terminal via USB serial connection at a baud rate of 9600. The setup proves the foundation process of data acquisition where processing data acquisition where physical rotational input is converted into data that high-level software can process. Whereas, in experiment 2, demonstrated control by transmitting user-defined angle commands from Python to actuate a servo motor.

Sources of Error and Limitations:

- Data Overflow: Sending the data too fast from the Arduino without a delay (e.g. delay(1000)) overwhelms the Python.

- Serial Port Conflict: The Arduino Serial Plotter and Python script cannot access the same COM port, opening both causes interrupted communication.

## 9.0 CONCLUSION

The experiments successfully demonstrated that serial communication (UART via USB) is a practical and effective bridge for linking an Arduino's hardware capabilities with Python's high-level processing. By maintaining a steady baud rate of 9600, we proved that data can flow reliably in both directions: pulling real-time analog readings from a potentiometer into a digital interface and pushing user commands back to actuate a servo motor.

The results fully supported our initial hypothesis. The servo motor moved precisely to the commanded angles and the potentiometer data allowed for smooth, live graphing through Matplotlib. This confirms that even simple serial protocols can handle the synchronization required for interactive mechatronic systems.

Beyond the lab, these findings have clear professional implications. The ability to translate physical sensor inputs into software logic is the core of Industrial Automation and Robotic Joint Control. Whether it is building a flight simulator HMI or an IoT monitoring system, the seamless integration of microcontrollers and high-level programming remains a fundamental skill in modern engineering.

## 10. RECOMMENDATIONS

- For future iterations, the circuit should be extended with an LED and a 220 $\Omega$ resistor to provide immediate visual feedback for logic thresholds, such as turning the LED ON when the potentiometer exceeds 50% of its range.

- The system can be improved by replacing the static "looping" servo code with real-time adjustments where the potentiometer directly dictates the servo angle via Python mediation.

- Students must ensure the Serial Plotter is closed before running Python scripts to maintain communication.

## 11. REFERENCES

- https://lastminuteengineers.com/servo-motor-arduino-tutorial/ How Servo Motor Works & Interface It With Arduino.

- https://www.instructables.com/Arduino-Servo-Motors/ Arduino Servo Motors

- https://makeabilitylab.github.io/physcomp/arduino/potentiometers.html

## 12. APPENDICES

Acknowledgements:

We would like to express our sincere gratitude to Dr. Zulkifli Bin Zainal Abidin and Dr. Wahju Sediono for their continuous support and for providing the essential resources and facilities required to carry out this project. We also extend our heartfelt appreciation to everyone who contributed to the success of this lab report through their valuable insights, assistance, and feedback.

Student’s Declaration:

We hereby affirm that the work presented in this report is entirely our own. All sources used for background information and theory have been properly cited, and  have strictly adhered to the principles of academic integrity throughout this report.